Standard BRB-seq Service

Sample Preparation and

Submission Guidelines

December 2025

MERCURIUS™

# BRB-seq for 96- and 384-well Plate Format

## Sample submission guidelines at a glance

- Transfer the RNA samples to a 96- or 384-well PCR plate following the instructions below and store them at -80°C before shipment.

- Fill in the Sample Submission Form (SSF) and check all the boxes in the Sample submission checklist below; send both files to orders@alitheagenomics.com. Please be aware that any inconsistency may lead to delays or additional fees.

- Request the shipping address from your sales specialist.

- Ship the samples on dry ice, ensuring the plates are placed between layers of dry ice to maintain a consistent freezing temperature throughout transit. Please provide us with the shipment tracking number.

## Sample submission checklist

- The Sample Submission Form (SSF) must be filled out correctly with a unique sample ID. Consider adding a suffix for technical replicates (e.g., XX_rep1, XX_rep2, etc.). Ensure that the SSF provides information about all the shipped samples

- Avoid having randomly distributed samples across the plate layout (e.g., in A01, A06, B03, G10-12, etc.).

- The sample volume should be 15 µL per well and identical for all samples (!).

- The concentration of all samples is uniform, and the A260/230 ratio is assessed and provided in the SSF.

- The minimum number of samples in each group (to be pooled together) is 16.

- Samples are provided in the 96-well RNase/DNase-free PCR plate. Samples in tubes cannot be processed.

- Plates are labeled with the same Plate ID as indicated in the SSF.

- Plates are well sealed with an adhesive and a temperature-resistant seal (aluminum is ideal).

## Essential considerations for input material

### Samples quantity and integrity

- The tested range of total RNA amount is 200 – 1000 ng (per well).

- The minimum volume should be 15 µL.

- The sample concentration should be at least 12 ng/µL.

- The recommended RIN number is > 6.

### Samples purity

- RNA samples extracted with TRIzol, phenol, chloroform, or guanidine are prone to residual contamination with organic solvents that considerably decrease cDNA yield.

- To ensure the high purity of RNA, assess the 260/230 ratio for all samples.

- The 260/230 ratio values should be between 1.8 and 2.2.

### Samples uniformity

- To ensure an even distribution of reads after sequencing, the RNA amount, integrity, and 260/230 values of the starting RNA samples must be as uniform as possible, with a max 10% variation.

- To obtain such uniform amounts, we recommend the following:

- Use dye-based methods for RNA quantification (e.g., Qubit Quant-iT or RiboGreen).

- Dilute samples to obtain the same RNA concentration in all wells (±10%).

- Re-measure the RNA concentration of all samples to confirm uniform concentration.

- Ensure the 260/230 ratio is between 1.8 and 2.2. and the RIN value is > 6 across the samples.

### Batch-effect and sample replicates

- RNA extraction protocol can produce considerable technical variation across the samples; therefore, performing RNA extraction in a single batch is strongly recommended.

- If the differential expression (DE) analysis is planned, the respective RNA samples should be included in the same library. Comparison of samples from different libraries can be biased.

- Including at least 3 (or more) biological replicates is highly recommended for the reliability of the experimental setup.

## Samples preparation

Label a new 96- or 384-well RNase/DNase-free PCR plate.

Pipette the RNA samples to the new 96- or 384-well PCR plate according to the filled Sample Submission Form. Follow the column-based direction (column 1, then column 2, etc.).

Seal the 96- or 384-well PCR plate with an aluminum seal and briefly spin it down.

Store the samples at -80°C before shipment.

Alithea Genomics SA

Phone

+41 78 830 31 39

Email

info@alitheagenomics.com

Web

www.alitheagenomics.com

Address

Route de la Corniche, 8

1066 Épalinges

VD, Switzerland

Phone

+41 78 830 31 39

Email

info@alitheagenomics.com

Web

www.alitheagenomics.com

Address

Route de la Corniche, 5

1066 Épalinges

VD, Switzerland